DMX Channel Instruction

| Channel Instuction |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel |  | Value | Description |  |
| 1 | Mode Setting | 0-9 | Laser OFF |  |
|  |  | 10-49 | PRG Mode |  |
|  |  | 50-99 | ILD Mode |  |
|  |  | 100-149 | Sound Active Mode |  |
|  |  | 150-199 | Auto Mode |  |
|  |  | 200-255 | Manual Mode |  |
| 2 | Pattern/Folder Select | 0-255 | Manual mode | PRG/ILD mode |
|  |  |  | Pattern select,every 3 value one pattern | File select |
| 3 | Flashing/File Select | 0-10 | No flashing | Play file select |
|  |  | 11-199 | Auto flashing |  |
|  |  | 200-255 | Audio flashing |  |
| 4 | X move | 0-125 | Adjust position by manual |  |
|  |  | 126-185 | Moving circle from left to right automatically |  |
|  |  | 186-225 | Jumping circle from left to right automatically |  |
|  |  | 226-245 | Auto jumping |  |
|  |  | 246-255 | Audio jumping |  |
| 5 | Y move | 0-125 | Adjust position by manual |  |
|  |  | 126-185 | Moving up and down automatically |  |
|  |  | 186-225 | Jumping up and down automatically |  |
|  |  | 226-245 | Auto jumping |  |
|  |  | 246-255 | Audio jumping |  |
| 6 | Zoom(+/-) | 0-10 | No change |  |
|  |  | 11-87 | Adjust size by manual |  |
|  |  | 88-150 | Zoom + |  |
|  |  | 151-200 | Zoom - |  |
|  |  | 201-255 | Zoom (+/-) circle |  |
| 7 | Rolling X | 0 | No rotation |  |
|  |  | 1-128 | Manual rotation |  |
|  |  | 129-255 | Auto rotation |  |
| 8 | Rolling Y | 0 | No rotation |  |
|  |  | 1-128 | Manual rotation |  |
|  |  | 129-255 | Auto rotation |  |
| 9 | Rolling Center | 0 | No rotation |  |
|  |  | 1-128 | Manual rotation |  |
|  |  | 129-292 | Auto clockwise rotation |  |
|  |  | 193-255 | Auto counterclockwise rotation |  |
| 10 | Drawing | 0-10 | No change |  |
|  |  | 10-74 | Manual drawing |  |
|  |  | 75-104 | Auto drawing + |  |
|  |  | 105-144 | Auto drawing - |  |
|  |  | 145-184 | Auto drawing circle |  |
|  |  | 185-224 | End to end drawing circle + |  |
|  |  | 225-255 | End to end drawing circle - |  |
| 11 | X wave | 0-9 | No wave |  |
|  |  | 10-69 | Small wave |  |
|  |  | 70-129 | Medium wave |  |
|  |  | 130-189 | Big wave |  |
|  |  | 190-255 | Biggest wave |  |
| 12 | Y wave | 0-9 | No wave |  |
|  |  | 10-69 | Small wave |  |
|  |  | 70-129 | Medium wave |  |
|  |  | 130-189 | Big wave |  |
|  |  | 190-255 | Biggest wave |  |
| 13 | Display Mode | 0-63 | Normal display |  |
|  |  | 64-127 | Light dot display |  |
|  |  | 128-191 | Segment display |  |
|  |  | 192-255 | Dot display |  |
| 14 | RGB Models Color Select | 0-16 | White |  |
|  |  | 17-33 | Red |  |
|  |  | 34-50 | Green |  |
|  |  | 51-67 | Blue |  |
|  |  | 68-84 | Yellow |  |
|  |  | 85-101 | Purple |  |
|  |  | 102-118 | Cyan |  |
|  |  | 119-135 | White,red,green,blue color section |  |
|  |  | 136-152 | Blue,yellow,purple,cyan color section |  |
|  |  | 153-169 | White,red,green,blue,yellow,purple,cyan 7 color section |  |
|  |  | 170-186 | White,red,green,blue 4 color flow |  |
|  |  | 187-203 | Blue,yellow,purple,cyan 4 color flow |  |
|  |  | 204-220 | Blue,yellow,purple,cyan 4 color flow |  |
|  |  | 221-237 | color subsection by inflexion |  |
|  |  | 238-255 | Sound active color change |  |
|  | RGY Model Color select | 0-27 | Yellow |  |
|  |  | 28-55 | Red |  |
|  |  | 56-83 | Green |  |
|  |  | 84-111 | Yellow |  |
|  |  | 112-139 | Red,green 2 color section |  |
|  |  | 140-167 | White,red,green,yellow 4 color section |  |
|  |  | 168-195 | Red and green color flow |  |
|  |  | 196-223 | White,red,green and yellow 4 color flow |  |
|  |  | 224-255 | Sound active change color |  |
| 15 | Grating select | 0-51 | No grating,normal animation effects |  |
|  |  | 52-102 | Animation multi-pattern effects |  |
|  |  | 103-154 | One dimension grating effects |  |
|  |  | 155-205 | 3D plane effects |  |
|  |  | 206-255 | 3D line effects |  |
| 16 | Red diode output power dimmer | 0-255 | Value 0 is brightest(100% output power), Value 255 is laser OFF(0% output) |  |
| 17 | Green diode output power dimmer | 0-255 | Value 0 is brightest(100% output power), Value 255 is laser OFF(0% output) |  |
| 18 | Blue diode output power dimmer | 0-255 | Value 0 is brightest(100% output power), Value 255 is laser OFF(0% output) |  |

1. The system just support short file name, file name (include folder name), most 8-bit file name and 3 extension name, file name and extension name formed by letter, number and under line. File name can’t over 8 and no Chinese words, otherwise the system can’t  identify it.

2. SD card can’t mix other file, most support 20 folders, every folder can save maximum 255 files and 10 PRG files.

3. Program table: user can use notepad to edit program list, the extension name is .PRG. Program table formed by play file name, play speed, play times, grating code(0: no grating, 1: Animation multi-pattern grating, 2: one dimension grating, 3: 3D plane grating, 4: 3D line grating. If no grating code, the default value is 0), ishow identify code "i"(if use ishow software make ild file), middle using“i”separate. Edit item, for example: one program formed by File1.ild, File2.ild, File3.ild. File1.ild play speed is 12K, replay 3 times, no grating effects;file2.ild play speed is 20K, play one time, animation multi-pattern effects, and file 2 is made by ishow software; file3.ild play speed is 18K, play 4 times, one dimension grating effects. Then prg1.prg contents as below:

File1.ild,12,3,0

File2.ild,20,1,1,i

File3.ild,18,4,2

When create one folder, need create the same PRG file under the folder, and put all the files need to play in the folder into the PRG file. For example, under aurora folder, create aurora.prg file.After adding ILDA file, need add the file to prg file, so that we can find the file quickly and play the file as per the speed you set.

Remote Operation